Appendix C: Implementation Details and Algorithmic Parameters

To ensure full transparency and reproducibility of the computational experiments, this appendix details the hyper-parameters, constraint handling mechanisms, and the computational environment utilized for the proposed M-NSGA-II framework.

C.1. Parameter Settings

The core hyper-parameters of the M-NSGA-II are summarized in Table C1. The crossover and mutation probabilities were determined through preliminary testing and kept consistent across all benchmark instances to evaluate the algorithm's generalized robustness without overfitting to specific problem sizes.

Table C1: Parameter Settings for the Proposed M-NSGA-II

| Parameter | Description | Value / Setting |
| --- | --- | --- |
| N | Population size | 100 |
| Genmax | Maximum generations (Stopping criteria) | 200 |
| pc | Batch-Aware Crossover probability | 0.90 |
| pm | Mutation probability (per chromosome) | 1/‖I‖ (where ‖I‖ is the number of parts) |
| pls | Break-Even Local Search probability (per offspring) | 0.10 |
| LSfrac | Fraction of parts evaluated during Local Search | 0.05 (5% of the total parts) |
| - | Selection mechanism | Binary Tournament Selection (comparing Rank and Crowding Distance) |
| - | Mutation operators | Swap, Relocate, or Orient (applied uniformly at random) |
| - | Elitism | Non-dominated sorting with Crowding Distance (Standard NSGA-II) |

Note: For the scalability stress tests on larger instances, the population size and maximum generations were dynamically scaled relative to the number of parts as N=max20,5×‖I‖ and Genmax=max2000,10×‖I‖, coupled with a stall-limit early stopping criterion to efficiently manage the computational budget.

C.2. Constraint Handling and Fitness Evaluation

Due to the heavily constrained nature of the 3D printing scheduling problem, traditional penalty functions often fail to guide the population effectively. Therefore, the proposed M-NSGA-II evaluates individuals using Deb’s Constrained Dominance Principle. Under this rule, any feasible solution strictly dominates any infeasible solution. If both solutions are infeasible, the one with the smaller cumulative constraint violation (capacity overflows) dominates. Objective functions (Ctot and CEtot) are only compared when both solutions are completely feasible or equally infeasible, ensuring that the evolutionary search pressure consistently drives the population toward the feasible regions of the solution space.

C.3. Computational Environment

All metaheuristic algorithms, including the baseline NSGA-II and the proposed M-NSGA-II, were implemented in Python 3.12. The exact MILP models were solved using the Gurobi Optimizer. All computational experiments and ablation studies were executed on a Lenovo ThinkPad T16 Gen 4 workstation equipped with an Intel Core Ultra 7 processor and 32 GB of RAM.