Appendix C: Implementation Details and Algorithmic Parameters

To ensure full transparency and reproducibility of the computational experiments, this appendix details the hyper-parameters, constraint handling mechanisms, and the computational environment utilized for the proposed M-NSGA-II framework.

C.1. Parameter Settings

The core hyper-parameters of the M-NSGA-II are summarized in Table C1. The crossover and mutation probabilities were set to the values commonly adopted for NSGA-II (Deb et al., 2002), verified through preliminary testing, and kept consistent across all benchmark instances to evaluate the algorithm's generalized robustness without overfitting to specific problem sizes.

When |I| is less than 20, 20% of the population is generated using the constructive heuristics. When |I| is 20 or higher, this percentage increases to 35%. These values were selected based on pilot tests with small and medium-sized instances. As the number of parts increases, fewer randomly generated chromosomes are feasible. Therefore, increasing the proportion of heuristic-generated chromosomes helps improve the feasibility of the initial population while maintaining diversity, since most chromosomes are still generated randomly. The threshold of |I| = 20 was set in the same pilot tests, and coincides with the largest instance size solved to proven optimality by the exact approach (see Section 6.2.1).

Each local search operation evaluates max(1, ⌊0.05 × |I| ⌋) parts, i.e., approximately 5% of the total number of parts. This choice follows common practice in lightweight memetic NSGA-II implementations (Ishibuchi et al., 2003), and was confirmed by preliminary tests, in which larger fractions provided only limited improvements in hypervolume while increasing the computational time.

The crossover probability (pc=0.9) and the mutation probability (pm=1/|I|) are the values commonly adopted for NSGA-II (Deb et al., 2002), with the mutation probability set to the reciprocal of the number of parts; they were also checked in preliminary tests on the 16-part instance used in the ablation study.

The local search probability (pls=0.10) for each offspring and the smart-operator probability (psmart=0.22) were selected during the same pilot testing phase. These values were not adjusted for different instance sizes. The only exception was the dynamic scaling of the population size and number of generations, which is explained below.

Table C1: Parameter Settings for the Proposed M-NSGA-II

| Parameter | Description | Value / Setting |
| --- | --- | --- |
| N | Population size | 100 |
| Genmax | Maximum generations (Stopping criteria) | 200 |
| pc | Batch-Aware Crossover probability | 0.90 |
| pm | Mutation probability (per chromosome) | 1/¦I¦ (where ¦I¦ is the number of parts) |
| pls | Break-Even Local Search probability (per offspring) | 0.10 |
| LSfrac | Fraction of parts evaluated during Local Search | 0.05 (5% of the total parts) |
| - | Selection mechanism | Binary Tournament Selection (comparing Rank and Crowding Distance) |
| - | Mutation operators | Swap, Relocate, or Orient (applied uniformly at random) |
| - | Elitism | Non-dominated sorting with Crowding Distance (Standard NSGA-II) |

Note: For the scalability stress tests on larger instances, the population size and maximum generations were dynamically scaled relative to the number of parts as N=max20,5×|I|  and Genmax=max2000,10×|I| , coupled with a stall-limit early stopping criterion to efficiently manage the computational budget. When the stall counter exceeds one-third of the stall limit (stall_limit / 3), pm, pls, and psmart are adaptively boosted by factors of 1.5 (capped at 0.30), 2.0 (capped at 0.50), and 1.4 (capped at 0.45), respectively.

C.2. Constraint Handling and Fitness Evaluation

Due to the heavily constrained nature of the 3D printing scheduling problem, traditional penalty functions often fail to guide the population effectively. Therefore, the proposed M-NSGA-II evaluates individuals using Deb’s Constrained Dominance Principle. Under this rule, any feasible solution strictly dominates any infeasible solution. If both solutions are infeasible, the one with the smaller cumulative constraint violation (capacity overflows) dominates. Objective functions (Ctot and CEtot) are only compared when both solutions are completely feasible or equally infeasible, ensuring that the evolutionary search pressure consistently drives the population toward the feasible regions of the solution space.

C.3. Computational Environment

All metaheuristic algorithms, including the baseline NSGA-II and the proposed M-NSGA-II, were implemented in Python 3.12. The exact MILP models were solved using the Gurobi Optimizer. All computational experiments and ablation studies were executed on a Lenovo ThinkPad T16 Gen 4 workstation equipped with an Intel Core Ultra 7 processor and 32 GB of RAM.

## References

Deb, K., Pratap, A., Agarwal, S., & Meyarivan, T. A. M. T. (2002). A fast and elitist multiobjective genetic algorithm: NSGA-II. IEEE transactions on evolutionary computation, 6(2), 182-197.

Ishibuchi, H., Yoshida, T., & Murata, T. (2003). Balance between genetic search and local search in memetic algorithms for multiobjective permutation flowshop scheduling. IEEE Transactions on Evolutionary Computation, 7(2), 204–223. https://doi.org/10.1109/TEVC.2003.810752